|  | National University of Computer and Emerging Sciences Chiniot-Faisalabad Campus |  |
| --- | --- | --- |

Name:__________________							Roll No:___________

Section:_________________							 Date:_____________

Quiz # 1

Question 1: Write a program to subtract 11 from 15 and then add 2 to it without using add, sub, mul, div .

# Sol:

[org 0x0100]

mov ax,15

xor ax,11

xor ax, 2

int 0x21

Question 2:  What is the difference between little endian and big endian formats? Explain with one example of each. Which format is used by the Intel 8086 microprocessor?

# Sol:

Little Endian

- In little endian format, the least significant byte (the "little end") is stored first, followed by the more significant bytes.

- This means that the lowest address contains the least significant byte.

- For example, if we have a 4-byte value 0x5678, it will be stored in memory as follows:

78 at (1000)

56 at (1002)

Big Endian:

- In big endian format, the most significant byte (the "big end") is stored first, followed by the less significant bytes.

- This means that the lowest address contains the most significant byte.

- Using the same example value 0x5678, it will be stored in memory as follows:

56 at (1000)

78 at (1002)

The Intel 8086 microprocessor uses the little endian format.

Question 3: Write the corresponding signed numbers represented by the following 4-bit system.

1000,1110, 1111.

# Sol:

1000 = -8, 1110 = -2, 1111 = -1.

Question 4: Give the value of carry flag, zero flag, sign flag and overflow flag after each of the following instructions are executed if AX = 0x1254 and BX = 0x0FFF.

- add ax, 0xEDAB

- add ax, bx

- add bx, 0xF001

# Sol:

- CF=0, ZF=0, SF= 1, OF=0.

- CF=1, ZF=0, SF= 1, OF=0.

- CF=1, ZF=0, SF= 1, OF=0.

Question 5: Why address bus is kept uni-directional?. Give a solid reason.

# Sol:

Address bus is kept unidirectional because memory is a dumb device and it cannot order or ask anything to the processor. Only processor can tell memory that which particular memory element is needed.

Question 6: Calculate the physical memory address generated by the following segments offsets pairs. Show working.

- FFFF:0001

- IDDD:4336

- 1060:5FF0

- AB01:01F0

# Sol:

- FFFF:0001              FFFF0+0001=FFFF1

- IDDD:4336	        IDDD0+4336=22106

- 1060:5FF0	        10600+5FF0=165F0

- AB01:01F0	         AB010+01F0=AB200