Assignment # 02

Computer Organization & Assembly Language

FALL-2023

Submission Guidelines

1. You must submit .pdf on the classroom having name 22F-XXXX.

2. Late submission is strictly not allowed.

3. Follow the deadline mentioned on classroom

4. Assignment must be on paper and in handwritten form.

5. Plagiarism is strictly prohibited, will result in 0 marks.

Question 1) Explain the difference between effective address and physical address? Give examples. (5 marks)

The 20 bit physical address is generated by adding 16-bit segment address and 16 bit effective address. The segment value is stored as a 20 bit number with the lower four bits zero and the offset value is stored as another 20 bit number with the upper four bits zeroed. The two are added to produce a 20bit absolute address. Example: IP=0100, CS=1DDD then PA= 1DDD0+00100=1DED

The effective address is the offset address which calculates the memory relative to a certain segment. In different addressing modes, the calculation of EA varies. For example, In register indirect addressing the offset address is held in any of the following registers: BP, BX, DI & SI. In base-indirect addressing, the offset address of the operand is computed by summing the base register to the contents of an Index register.

Question 2) Update registers after executing following code (5 marks)

mov al,[val1]

mov bx,[val2]

add bx,ax

Mov cl,[val3]

Add ax,cx

Add bx,ax

mov [result],bx

mov dl,bl

mov cx,[result]

val1: db 5

val2: dw 012h

Val3: db 2

result: dw 0

| AX | 0007 |
| --- | --- |
| BX | 001E |
| CX | 001E |
| DX | 001E |

Question 3) Given the following data (10 marks)

num1: db ‘8’

num2: db 64,65h,’z’,11101111b

num3: dw –1

num4: db “abcd”,0

num5: dw 10,20,30,40,50

num6: dw 12345678,11223344,90876543

Update the SI register after every instruction. Take the 3rd,4th,5th and 6th digit of your roll number. Consider 0x<3rd,4th ,5th,6th> to be the starting address. Indicate address as 0x___

(if roll num: 22F-1234, then 3rd=1, 4th=2, 5th=3, 6th=4)

mov si,offset[num1+4]

mov si,offset num4

mov si,offset [num5+4]

mov si,offset num3

mov si,offset[num6+10]

mov si,offset [num2+3]

mov si,offset [num3+12]

If roll num=22F-1234 then, starting address is 0x1234. This is the mapped address

| Num1: db is 1 bytes | 0x1234 |
| --- | --- |
| Num2: db is 4 bytes | 0x1238 |
| Num3: dw is 2 bytes | 0x123A |
| Num4:db is 5 bytes | 0x123F |
| Num5:dw is 10 bytes | 0x124F |
| Num6:dw is 6 bytes | 0x1255 |
| Mov si,offset[num1+4] | 0x1238 |
| Mov si,offset num4 | 0x123F |
| Mov si,offset [num5+4] | 0x1253 |
| Mov si,offset num3 | 0x123A |
| Mov si,offset [num6+10] | Overflow address |
| Mov si,offset [num2+3] | 0x123B |
| Mov si,offset [num3+12] | 0x124C |

Question 4) Explain the difference between short jump and far jump with examples

Short Jump

If the offset is stored in a single byte as in 75F2 with the opcode 75 and operand F2, the jump is called a short jump. F2 is added to IP as a signed byte. If the byte is negative the complement is negated from IP otherwise the byte is added. Unconditional jumps can be short, near, and far. The far type is yet to be discussed. Conditional jumps can only be short. A short jump can go +127 bytes ahead in code and -128 bytes backwards and no more. This is the limitation of a byte in singed representation.

Far Jump:

Far jump is not position relative but is absolute. Both segment and offset must be given to a far jump. The previous two jumps were used to jump within a segment. Sometimes we may need to go from one code segment to another, and near and short jumps cannot take us there. Far jump must be used and a two byte segment and a two byte offset are given to it. It loads CS wit the segment part and IP with the offset part. Execution therefore resumes from that location in physical memory. The three instructions that have a far form are JMP, CALL, and RET, are related to program control. Far capability makes intra segment control possible.

Question 5) Sort the given list using bubble sort in ascending order (Give dry run and attach screenshots of afd debugger, otherwise marks will be deducted) (25 marks)

arr: dw  24,-56,52,00,12,60,48,34,16,02

swap: db 0

[org 0x0100] ;m2 ds:add

swap1:

mov bx, 0

mov byte [swap], 0

l1:

mov ax, [num+bx]

cmp ax, [num+bx+2]

jl l2

mov dx, [num+bx+2]

mov [num+bx+2], ax

mov [num+bx], dx

mov byte [swap], 1

l2:

add bx, 2

cmp bx, 18

jne l1

cmp byte [swap], 1

je swap1

mov ah, 0x4c

int 0x21

Question 6) Write a program to check whether an integer is positive, negative or zero. Give written dry run and attach screenshots of afd debugger, otherwise marks will be deducted. (5 marks)

org 0x100

mov ax, [num]

cmp ax, 0

jz zero

js negative

positive:

mov byte[pos], 1

jmp exit

negative:

mov byte[negt],1

jmp exit

zero:

mov byte[zeroo], 1

exit:

mov ax,4ch

int 21h

num: dw 1

pos: db 0

negt: db 0

zeroo: db 0