National University

of Computer and Emerging Sciences, Chiniot-Faisalabad Campus

Name:                              COAL                                    Roll No:

Secton:                           Quiz#02                                  Date:

Question 1) Give concise reasoning with examples. (3 marks)

- Can a destination operand be less than or greater than the source operand?

The source and destination operands must have the same size because intel specified it as such.

- Is mov ax,[bl+si] valid?

BX is allowed to hold an address but BL or BH are not. Address is 16bit and must be contained in a 16bit register

- Is BX-BP and BX+BP possible?

The only thing that we can do is addition of a base register with an index register. Any other operation is disallowed.

Bx+BP isnt allowed because one has to be a base register and the other has to be an index register and that is the reason of naming them differently.

- If an array is of double-word (32 bit) size, what should be the offset to move to next index.

The offset would be of 4 because the array is of 32bit/4 bytes, hence each element would take up 4 bytes.

- Can SP access the data segment?

SP is tied to the stack segment and cannot be overriden to access data segment

- Can physical address be formed with the help of effective address?

Yes, effective address if BX=9100, DS=1500 and the access is [bx+0x7000] we form the

effective address 9100 + 7000 = 10100 and the physical address is then 15100

Question 2) Write the registers that are associated with each segment by default. Can we override all of them? If not, then why?  (1.5 marks)

CS-IP

DS- BX, SI, DI

SS- BP,SP

No mention of ES segment in readings so no need to check that.

We cannot override all because some registers are tied down to specific segments. Like IP is tied to CS and SP is tied to SS.

Question 4) Which segment will the register access in the following instructions? And why? (1 marks)

- Mov ax,[bx+di]

- Mov ax,[bx+si]

In both cases data segment will be accessed because default segment association is based on the base register.

Question 05) Write the value of ax and bx register after execution of each line [7 marks]

[org 0x0100]

mov bx,num1

mov ax,[bx]

add bx,2

add ax,[bx]

add bx,2

add ax,[bx]

add bx,2

mov [bx],ax

mov ax,04c00

int 0x21

num1:dw 5,10,15,0

| mov bx,[num1] | address of 5/address of first element of num1 |
| --- | --- |
| mov ax,[bx] | ax=5 |
| add bx,2 | address of 10 |
| add ax,[bx] | ax=5+10=15 |
| add bx,2 | address of 15 |
| add ax,[bx] | ax=15+15=30 |
| add bx,2 | address of 0 |
| mov [bx],ax | bx=30 |